# Unit 0 — Before the Machine · Answer Key (Instructor Copy)

Model answers with mark allocation and Bloom's taxonomy levels. Pattern: Part A = Remember/Understand (1 mark); Part B = Understand/Apply (2 marks); Part C = Apply/Analyze/Evaluate (5 marks).

## Part A — 1-mark questions

Q1. Define a bit.   [1 mark · Bloom's: Remember]

Answer: A bit (binary digit) is the smallest unit of information, taking exactly one of two values, 0 or 1 — physically represented by two distinct voltage levels.

Q2. Define computer architecture.   [1 mark · Bloom's: Remember]

Answer: Computer architecture is the structure and behaviour of a computer as seen by the user/programmer — the instruction set, data formats and addressing techniques (the 'what').

Q3. Define computer organization.   [1 mark · Bloom's: Remember]

Answer: Computer organization is the set of operational units and their interconnections that implement the architectural specification — circuits, control signals and technology (the 'how').

Q4. What voltage levels typically represent logic 0 and logic 1?   [1 mark · Bloom's: Remember]

Answer: Logic 0 = low voltage, approximately 0 V (ground); logic 1 = high voltage, typically +5 V or +3 V.

Q5. State the stored-program (von Neumann) concept.   [1 mark · Bloom's: Remember]

Answer: Instructions are stored in main memory alongside data, encoded as numbers; the machine runs a new program by loading new numbers — no rewiring.

Q6. List the four generations of computer technology.   [1 mark · Bloom's: Remember]

Answer: 1st: vacuum tubes (1945–55); 2nd: transistors (1955–65); 3rd: integrated circuits (1965–75); 4th: VLSI (1975–present).

Q7. What is the role of the operating system?   [1 mark · Bloom's: Remember]

Answer: The OS is a program that manages the computer's resources and handles the interaction between application software and the hardware.

Q8. What motivated Charles Babbage to design the Difference Engine?   [1 mark · Bloom's: Remember]

Answer: The many errors in hand-computed, hand-typeset printed tables (logarithmic, astronomical/navigation, actuarial) used in the 1800s — he wanted a machine to compute AND print them without human error.

Q9. What does an XOR gate output when both inputs are equal?   [1 mark · Bloom's: Understand]

Answer: 0. XOR outputs 1 only when its inputs DIFFER; equal inputs (0,0 or 1,1) give 0.

## Part B — 2-mark questions

Q1. Differentiate between computer architecture and computer organization, giving one example of each.   [2 marks · Bloom's: Understand]

Answer: Architecture: programmer's view — e.g. the instruction set (ISA), data formats, addressing modes. Organization: implementer's view — e.g. the electronic circuits, control signals, CMOS technology realising those instructions. [1 mark per side with a valid example.]

Q2. Convert the binary number 1011 to decimal, showing the positional weights used.   [2 marks · Bloom's: Apply]

Answer: Weights: 8 4 2 1. 1011 = (1×8) + (0×4) + (1×2) + (1×1) = 8 + 0 + 2 + 1 = 11. [1 mark working, 1 mark answer.]

Q3. What is the forbidden voltage range, and why is it necessary?   [2 marks · Bloom's: Understand]

Answer: The band of voltages between the maximum recognised as logic 0 and the minimum recognised as logic 1. Signals must not settle there: it guarantees that random electrical noise, which can wiggle a voltage slightly, cannot push a 0 across the gap to be misread as a 1 (or vice versa) — the source of binary's reliability. [1 mark definition, 1 mark noise reasoning.]

Q4. How is a NAND gate built from transistors, and why does the series connection produce NAND?   [2 marks · Bloom's: Understand]

Answer: Two transistors are connected in SERIES between the output and ground, with a resistor to the supply. Only when BOTH inputs are high do both transistors conduct, pulling the output low; any other combination leaves the output high — exactly NAND. [1 mark construction, 1 mark reasoning.]

Q5. State the power advantage of CMOS technology and its cause.   [2 marks · Bloom's: Understand]

Answer: CMOS pairs complementary NMOS and PMOS transistors so that in either steady state one of the pair is off — almost no current flows at rest; power is consumed mainly during switching. This is why chips with billions of gates remain thermally manageable. [1 mark advantage, 1 mark cause.]

Q6. The need for automatic computation arose twice in history for similar reasons. Compare Babbage's motivation (1821) with the motivation that funded ENIAC (1940s).   [2 marks · Bloom's: Understand]

Answer: Both stemmed from error-prone human computation of printed tables. Babbage (1821): errors in mathematical/navigation/actuarial tables caused shipwrecks and financial losses — he designed the Difference Engine to compute and print tables mechanically. ENIAC (WWII): artillery firing tables (~50 hand calculations per entry) were produced too slowly and with errors by human computers — the US Army funded an electronic machine. Same disease, a century apart. [1 mark per motivation.]

## Part C — 5-mark questions

Q1. Distinguish between computer architecture and computer organization. Support your answer with the house analogy, a table of at least three differences, and the example of a 1970s program running on a modern processor.   [5 marks · Bloom's: Analyze]

Answer: Definitions: architecture = structure and behaviour as seen by the programmer (ISA, data formats, addressing); organization = operational units and interconnections implementing it (circuits, control signals, technology). Analogy: architecture is the house blueprint (rooms, socket positions); organization is the wiring and plumbing behind the walls. Table: view (programmer vs implementer); concerns (what instructions exist vs how they are executed); components (ISA/formats/addressing vs circuits/control signals/CMOS); stability (changes rarely vs changes with every technology generation). Example: a program written against a 1970s ISA can run unchanged, a billion times faster, on a modern VLSI implementation — the architecture survived while the organization was rebuilt many times. [Definitions 1; analogy 1; table 2; example 1.]

Q2. Design a half adder: give its truth table, Boolean expressions, and gate-level diagram. Explain why XOR produces the sum and AND produces the carry.   [5 marks · Bloom's: Apply / Analyze]

Answer: Truth table (A B → Carry Sum): 00→00, 01→01, 10→01, 11→10. Expressions: Sum = A ⊕ B; Carry = A · B. Diagram: inputs A and B fan out to one XOR gate (output Sum) and one AND gate (output Carry). Why: in binary addition the sum bit is 1 exactly when the inputs differ (0+1, 1+0) — the XOR function; the carry is 1 only when both inputs are 1 (1+1 = 10) — the AND function. [Truth table 2; expressions 1; diagram 1; explanation 1.]

Q3. Compare the relay, the vacuum tube and the transistor as switching devices, and justify why the transistor made modern computing possible.   [5 marks · Bloom's: Evaluate]

Answer: Relay: electromagnet physically moves a contact arm; ~1 kHz; noisy, mechanical wear. Vacuum tube: grid voltage steers electrons from a heated cathode; ~1 MHz; no moving parts but hot, power-hungry, burns out (ENIAC: 18,000 tubes, ~1 failure/day). Transistor (Bell Labs, 1947): solid-state switching in silicon; no filament, no moving parts; small, cool, reliable. Justification: only the transistor can be miniaturised and mass-fabricated — integrated circuits grow billions of switches together on silicon (2,300 in the 4004 → 100 B+ today), delivering the speed, density, reliability and cost that relays and tubes fundamentally cannot reach. [Comparison 3 — one per device; justification 2.]

Q4. Describe the four generations of computer technology and explain Moore's law, including why it held for decades and why it is now slowing.   [5 marks · Bloom's: Understand / Analyze]

Answer: Generations: 1st (1945–55) vacuum tubes — stored-program concept, assembly language; 2nd (1955–65) transistors — smaller/faster gates, high-level languages (Fortran); 3rd (1965–75) integrated circuits — many transistors per chip, pipelining and cache concepts emerge; 4th (1975–present) VLSI — billions of transistors, the microprocessor. Moore's law: the observation (Gordon Moore, 1965) that transistor count per chip doubles roughly every two years — exponential growth, a straight line on a log scale (1 → 2,300 → 1.2 M → ~1 B → 100 B+). It held because IC fabrication photographs circuits onto silicon — shrinking the feature size multiplies the count at little extra cost, and industry investment chased that target. It slows as features approach atomic scale (~2 nm): quantum leakage, heat density and fabrication cost rise sharply — shifting progress toward better ARCHITECTURE rather than smaller switches. [Generations 2; Moore statement 1; why it held 1; why slowing 1.]